Table of Contents

# Electrical Drawing Interpretation Pipeline — Technical Documentation

Pratik Shrivastava

## 1. What this is

The task was to take a raster image of an electrical drawing and read it the way an engineer would: find every symbol, work out what each one is, figure out how they’re wired together, and hand back a structured netlist plus a graph. Five drawings were provided. Two of them (D4, D5) are electronic circuit schematics — resistors, transistors, a MOSFET, an optocoupler. The other three (D1, D2, D3) are process & instrumentation diagrams: vessels, centrifugal pumps, control valves, instrument bubbles, heat exchangers. Same brief, genuinely different symbol vocabularies and drawing conventions.

The first version of this pipeline only handled the two schematics, because that’s what I started looking at. The bulk of the interesting engineering happened when I went back and made it handle both kinds of drawing without rewriting the pipeline — and that restructuring is what most of this document is about, because it’s the part I’d actually defend in a review.

The brief is explicit that raw accuracy isn’t the main thing being scored — it’s whether the approach is generic and scalable, and whether every decision along the way can be justified. I took that seriously. A lot of what follows is me explaining why I didn’t do the thing that would have bumped a number.

Running it: conda env create -f environment.yml, pip install -e ., bash scripts/bootstrap.sh, then edp run data/validation/D4.png --out outputs/ or edp serve. The shipped model weights and all symbol assets are in the repo; retraining from scratch is documented in README.md. The design history and every measured experiment referenced below are in docs/08–docs/13.

## 2. The core idea: separate the machinery from the knowledge

Here’s the problem I kept running into. A pipeline stage like “binarize the image” or “skeletonize the wires” or “fuse the classifier votes” is identical whether you’re looking at a circuit or a P&ID. But “what does a resistor look like”, “what does the letter R mean on a label”, “is a dot on a crossing a connection” — those are drawing-type-specific, and in the first version they were scattered through the stage code as hardcoded paths, magic strings and if-branches. So the only way to support a P&ID was to go edit the machinery, which felt wrong, because the machinery wasn’t the part that needed to change.

The fix was to pull all the drawing-type-specific knowledge into a thing I call a domain pack — a folder plus a small config object. Everything the stages consult that varies by drawing type lives in the pack:

| what varies | electronic pack | P&ID pack |
| --- | --- | --- |
| which trained detector to run | KiCad-synthetic YOLO, 17 classes | ISA-synthetic YOLO, 10 classes |
| the reference symbol library | KiCad-rendered symbols | procedurally-drawn ISA symbols |
| the OCR text prior | R, C, ZD… (IEC 61346) | PC, LT, FCV… (ISA-5.1) |
| the linear-probe classifier | trained artifact | none |
| geometry specialists | battery/capacitor, BJT/potentiometer | none yet (stubbed, pairs named) |
| evidence-fusion weights | YOLO gets a modest edge | OCR gets the edge (ISA tags are diagnostic) |

The machinery — preprocessing, localization, the evidence-fusion classifier, OCR association, the connectivity graph, the JSON and graph output, the evaluation harness — is completely domain-agnostic. When I added the P&ID pack, not one line of preprocess.py, classify/match.py, ocr.py, wires.py or emit.py changed. The pipeline orchestrator gained one new stage (routing, below) and started passing a pack object into two function calls. That was the entire diff to the machinery.

I think of it like a spell-checker. The engine that finds word boundaries, compares against a list, underlines the misses and ranks suggestions is the same for every language. The word list and grammar rules are a per-language pack. Adding Spanish support doesn’t mean rewriting the spell-checker. That’s the property the brief is asking for when it says “generic and scalable”, and building the P&ID pack was me proving the pipeline actually has it, rather than just claiming it.

Concretely, a pack is edp/domains/<name>/pack.yaml — a few paths and weights — plus, next to it, the small knowledge files that pack references (a specialists.py, a designators.yaml or tags.yaml). Large regeneratable assets — the reference image libraries, the trained weights — stay under data/ and are pointed at by path. edp/domains/base.py loads a pack and exposes it as a frozen dataclass. Adding a third drawing type later — a single-line power diagram, say — is a new folder. No core code.

## 3. Walking through the pipeline

An image comes in and goes through eight stages. edp/pipeline.py is the only file that knows the full order and why it’s that order; every stage module takes typed objects in and returns typed objects out, with no knowledge of what ran before or after it. That isolation is deliberate — when something goes wrong I want exactly one place it could have gone wrong.

### Preprocess

Grayscale, then an adaptive (local-mean) threshold rather than one global cutoff, because contrast isn’t always even across a page and I didn’t want a threshold I’d have to retune per drawing. Then a small-angle deskew (these are digitally-generated drawings, not scanned pages, so I’m correcting a few degrees of export tilt, not building rotation invariance). Then speckle removal — and this one bit me. My first instinct was a morphological opening. I rendered the binarized output and watched it erase almost every one-pixel-wide wire and symbol line in the drawing, because a line that thin can’t contain even a tiny structuring element anywhere along its length. The junction dots survived; the actual circuit didn’t. Switched to connected-component area filtering, which judges a blob by its size, not its width, so a long thin wire and a fleck of noise get treated differently.

One drawing (D5) draws some conductors in blue. There’s an optional colour mask that pulls blue ink out as a separate signal. It’s explicitly opportunistic — on a monochrome drawing it returns empty, and everything downstream is written to treat “no colour signal” as normal, not an error, because half the input has no colour.

### Route

New stage. If the domain is set to auto (the default), the pipeline decides whether this is a circuit or a P&ID from the drawing’s text alone. Each pack contributes its designator prefixes plus a keyword list — circuits have OHM, UF, VDC, part numbers like BC547; P&IDs have GPM, OUTLET, CONDENSATE, VESSEL, and ISA instrument tags. The OCR tokens are scored against both packs, higher wins, and a thin or tied signal falls back to a configured default so a text-sparse drawing still runs. OCR runs once here and the tokens are handed to the rest of the pipeline so it isn’t run twice.

I tried this on all five drawings. D1/D2/D3 route to P&ID with scores of 19/23/76 against 1/5/0 for electronic; D4/D5 route to electronic 61/17 against 18/3. Wide margins, no ambiguity. It’s a cheap, explainable rule — no model, no training — and it works because the two kinds of drawing genuinely say different things on them.

### Localize

The shipped path is a YOLOv8n detector, and I generated all of its training data myself. There’s a classical density-based localizer as a fallback if no trained weights are present, so the pipeline stays runnable before any model exists — that “walking skeleton” discipline is threaded through the whole codebase (empty reference library, missing probe artifact, missing detector all degrade rather than crash).

The classical localizer was the original approach — connected-component analysis over skeleton branch/endpoint density. I shipped it early because it needs no training data. It fell apart when I looked at the output: roughly six out of ten boxes were wrong, mostly wire bends and rail junctions that looked “symbol-shaped enough” to a density heuristic, and it missed real components too. I could have kept tuning thresholds but I’d have been chasing noise. So I trained a proper detector — YOLO, because it learns actual symbol shapes and the nano variant is small enough to retrain in minutes on a laptop GPU whenever the reference library changes, which mattered given how many iterations this went through.

How the training data is made: KiCad-rendered reference symbols get pasted onto blank canvases at the real drawings’ size distribution (measured from D4/D5, not guessed), rotated 0/90/180/270, with synthetic Manhattan-routed wire clutter and dash-cluster text clutter added as unlabelled negatives — so the detector learns not to fire on wires and text without ever being told “this is not a symbol”, it just never sees a label there. The bounding-box label is a byproduct of where the generator put each symbol. Nobody drew a box. The shipped model mixes three generators: sparse scatter, dense packing, and a topology-realistic “ladder” generator with rails and branches and junction dots, built as a separate third generator specifically so a low score would be attributable — “doesn’t handle real topology” versus “doesn’t generalize to unfamiliar icon styles” are different problems and one harder scatter generator would have conflated them.

The P&ID detector is the same story with a different symbol source (§8).

### Classify

This is the part I spent the most time on and ended up furthest from my first instinct. Details in §4.2. The short version: every candidate box gets voted on by up to five independent sources, weighted by each source’s own confidence in this particular reading, and any source is allowed to abstain rather than guess.

### Text association

Tesseract, run across three page-segmentation modes and three orientations per crop, keeping every reading without deduplicating. Details in §5. The output resolves instance identity (which resistor is R1 versus R2) and reads component values.

### Wires

Subtract the classified symbol ink from the binary image (keeping wires that merely pass through a box), skeletonize what’s left to one-pixel-wide lines, re-place each symbol’s terminals onto the point where a wire actually touches its bounding box, build a pixel graph, and walk it into nets. The one rule that does real work here: a wire crossing is only a connection if there’s a dot drawn on it. Details in §6.

### Validate

Self-consistency checks — symbols with terminals but no net, terminals that never snapped to anything, nets with only one member. Not an accuracy score (there’s no ground truth for an arbitrary new drawing); it’s there to catch structural nonsense before it reaches the output.

### Emit

The JSON matches the schema in the brief exactly — id, type, coordinates, connections, nothing decorative. The graph is built bipartite internally (symbol nodes and net nodes as distinct types) because a five-terminal net expanded straight into a symbol-to-symbol graph becomes a ten-edge clique implying wires that don’t exist. The delivered component graph is projected from the same trimmed JSON the file output uses, not recomputed independently, so the picture and the data can’t silently disagree.

## 4. Model selection rationale

### 4.1 Localization — YOLOv8n

I went with YOLOv8n over the classical-only approach because the classical one had a measured ~60% false-positive rate on D4 and no notion of what a resistor should look like versus noise. YOLO gave me that, and the nano variant (about 3M parameters) trains in minutes on a 4GB laptop GPU. The trade-off I accepted going in, and which held for the whole project, is that it’s only as good as its synthetic training data — it never saw a real drawing until evaluation, so its recall on unusual real-world symbol styles is the weakest link in the pipeline. Two attempts to fix that with more or different training data both regressed and were reverted (§10).

I did not use YOLO’s own class predictions as final. It’s trained purely on synthetic composites, and fine-grained class distinctions — NPN versus PNP, diode versus zener — are exactly where the synthetic-to-real gap bites hardest. So YOLO’s class and confidence go in as one vote among five, not the answer.

### 4.2 Classification — five-source evidence fusion

I started with one method: DINOv2 embeddings matched against a reference library by nearest neighbour. No labelled training data, just example crops of each symbol type. It worked, but it made confident, silent mistakes on visually similar symbols — capacitor versus battery, BJT versus potentiometer — and I had no way to know when it was guessing versus sure.

My next instinct was to let YOLO only localize and have DINOv2 name everything, since I didn’t fully trust YOLO’s synthetic-trained class head. But when I compared the two side by side on D4, they were wrong on completely different symbols. YOLO got about 10 of 16 right, DINOv2 about 7 of 16, and the errors barely overlapped. Discarding either one was throwing away real information. So classification stopped being one model’s call and became a weighted vote.

Each source produces a ClassificationEvidence: a dict of {class: score}, a scalar confidence in this specific reading, and some metadata for explainability. Or it produces an explicit “no evidence” — abstaining, not guessing. Fusion sums, per class:

total[class] += weight[source] × evidence.confidence × evidence.class_scores[class]

then takes the argmax. Two things this buys over a fixed priority order like “OCR beats DINOv2 beats YOLO”:

- A source’s own certainty about this reading matters, not just a fixed per-source weight. A garbled OCR read contributes almost nothing even though OCR as a source has real weight.

- Two sources agreeing pushes a class up additively. Agreement is rewarded structurally, not with a special-cased rule.

The five sources:

- YOLO’s class head — {yolo_class: 1.0} at the detector’s own confidence. Never abstains, but a low-confidence box contributes almost nothing.

- DINOv2 nearest-neighbour — each candidate crop is embedded and matched by cosine similarity against a library of embedded reference symbols. Returns the top-3 distinct classes, over-fetching 20 raw neighbours first so many rotated copies of one class don’t crowd out a genuinely different second candidate. Below a similarity threshold it abstains entirely — a weak match is worse than no match, because it would vote confidently for a wrong class.

- Why DINOv2 and not CLIP: DINOv2 is trained for dense visual similarity, which is better for near-duplicate shape matching of technical line art. CLIP is trained for image-text alignment and is measurably weaker at telling a resistor’s zigzag from an inductor’s coil, because in its image-text space those are “similar enough”.

- Why nearest-neighbour and not a fine-tuned classifier: this is the single decision that makes the system generic in the sense the brief asks for. Adding a symbol class is dropping in reference crops and rebuilding an index — no retraining, no new labelled dataset, no GPU time. A fine-tuned classifier would need retraining (with the regression risk demonstrated twice in §10) every time the class set changes.

- DINOv2 embeddings aren’t rotation-invariant, so the library is augmented at build time with four rotations and a mirror, and the matched rotation is kept — it’s what orients the terminal template onto the candidate box later.

- Linear probe — a scikit-learn logistic regression trained on frozen DINOv2 embeddings of domain-randomised synthetic crops. A learned decision boundary over the same embedding space the nearest-neighbour match uses, built as an independent hypothesis: does a learned boundary generalize better than “closest single reference point”? Kept as a genuinely separate vote rather than a replacement — if it’s worse on some case it can only add a wrong vote that a correct nearest-neighbour vote outweighs, never override a good source. Reuses the embedding already computed for source 2, so it’s nearly free.

- OCR reference-designator / part-number prior — engineering designators are a documented convention (IEC 61346, ANSI Y32.2): R for resistor, C for capacitor, ZD for zener. Specific part numbers are stronger still — BC547 is unambiguously an NPN transistor. This is knowledge no vision model has. Three confidence tiers: exact part number is near-authoritative (0.95), a clean designator is a moderate vote (0.55), anything unparseable contributes nothing at all. That last part is a deliberate rule — never let garbled OCR outvote confident visual evidence by treating noise as weak-but-nonzero. The table is a YAML file, not code; adding a convention is a data edit. Some designators are deliberately absent — T means Transformer in one drawing and Transistor in another, so a prior there would be wrong half the time by the convention it’s meant to encode.

- For the P&ID pack this same mechanism carries the ISA-5.1 tag convention: first letter is the measured variable (P, L, F, T), succeeding letters are the function (I, C, T, V), every such tag is drawn as the same bubble so they all map to the Instrument class.

- Geometry specialists — hand-coded checks that only run when fusion is already ambiguous between classes in a known confusion group. Battery versus capacitor is decided by plate count and curvature; BJT versus potentiometer by whether there’s a circle. Hand-coding is right here specifically because the distinguishing feature is a documented drawing convention, not a learned statistical pattern.

- One specialist — NPN versus PNP by arrowhead direction — is built, unit-tested, and deliberately not wired in. It’s 2/2 correct on clean reference renders but 1/3 on real D4 crops, worse than chance. I left it in the module, documented, and excluded from routing rather than shipping it anyway. That’s one of the stronger “why should I trust your judgment” answers I have — the same measure-before-trusting discipline applied even to work that was fully built and worked in principle.

The default fusion weights (YOLO 1.2, everything else 1.0) are a single reasoned default, not a grid search. The reasoning — YOLO is the supervised, in-domain signal so it gets a modest, not dominant, edge — is real and was checked for not regressing anything. The specific value 1.2 wasn’t swept. That’s a genuine gap; the fusion architecture is the justified decision, the exact weight is a reasonable default that hasn’t hurt.

### 4.3 OCR — Tesseract, multi-pass

Deterministic, dependency-light, adequate on clean synthetic-quality glyphs once upscaled. I upscale 3× with Lanczos first, because D5’s labels are only a few pixels tall natively and Tesseract has an effective floor around 20px of glyph height. That’s the single highest-impact OCR decision. Honest caveat: upscaling reconstructs, it doesn’t add information — a genuinely sub-5px glyph stays unreliable.

My first version ran Tesseract once per region and missed anything rotated or anything its default mode wasn’t tuned for. Now it runs three PSM configurations across three orientations and keeps every reading without deduplicating. I measured every PSM × upscale combination against known designators on both drawings first — no single config won on both, and merging all three beat every individual config. I tried deduplicating by keeping the highest-confidence reading per region and it lost a real answer (a correct “$1”→“S1” switch designator crowded out by a higher-confidence but wrong reading from a different pass) — Tesseract’s confidence isn’t a reliable “which reading is right” signal for short garbled technical tokens. So I pass everything through and let downstream nearest-token matching surface the correct one redundantly.

### 4.4 Connectivity — classical CV, not a learned model

These drawings are thin, high-contrast, mostly-orthogonal strokes — the textbook case for skeletonization. No training data exists for a learned segmentation model and none is needed for this input. A classical approach stays fully deterministic and traceable to a specific pixel rule, which matters for the same “justify every decision” reason as everything else. I ruled out detecting wires as another YOLO class early — wires aren’t compact objects, they’re long thin structures of arbitrary shape, and a bounding box is the wrong representation.

### 4.5 Where a VLM or LLM was, and wasn’t, used

Not in the pipeline, and that was deliberate. Early on I looked at prompting a vision-language model to read the whole drawing and output structured symbols and connections directly. It’s tempting because it needs almost no engineering. But I couldn’t get it to a place where I trusted the output — no clear notion of confidence, no way to know why it made a call, no way to systematically improve it when it got something wrong beyond rewriting the prompt and hoping. The brief also explicitly lists preprocessing logic, detection strategy, connectivity inference, post-processing and validation, JSON generation and graph construction as things I must develop — a VLM call that returns a finished description doesn’t produce any of those as inspectable stages, and all six would still need to exist and be justified around it.

Where I did use an LLM: authoring the reference-designator and ISA-tag YAML tables once, at build time, as a documentation aid. The shipped system reads a static file on every request, never a model. If asked “did you use an LLM” the honest answer is yes, as an authoring aid, never as a runtime dependency.

The place I’d actually bring an LLM in, if I extended this, is post-hoc — a sanity-check layer reviewing the final JSON for an implausible component value or a mislabelled net, not the core extraction. That’s a real scoped next step, not a rejected idea; it’s just not warranted until there’s evidence the deterministic methods have plateaued.

## 5. Pre-processing and post-processing

Preprocessing is covered in §3. The one thing worth repeating is the denoise-by-component-area decision, because a morphological open — the obvious choice — actively destroys these drawings, and I only caught it by rendering the intermediate output and looking.

On the post-processing side:

Duplicate merging. Both localizers occasionally produce two or three boxes for one physical symbol — the density one fragments, and YOLO’s own NMS doesn’t catch every near-duplicate from different anchor scales. I merge any pair of boxes whose overlap passes a threshold, but I don’t blindly discard the loser — I keep whichever duplicate had the higher classification confidence, because I found a real case where two duplicate boxes for the same transistor scored two different classes and the more confident one was correct. When the real-data detector (§10) is enabled, this same merge is what folds its class-agnostic boxes in alongside YOLO’s class-bearing ones.

Consistency checks. After the pipeline runs, I check the result against itself — is every symbol with terminals actually on a net, is every terminal accounted for, is any net down to a single member. None of this is against ground truth. It’s a self-consistency pass to catch structural nonsense that would embarrass the output even if I never saw the source image.

Confidence and abstention. Every symbol carries the fused confidence plus a margin (top guess minus runner-up, on a normalised scale). A low margin is the system saying “I chose this but it was close”. Symbols under a minimum confidence get flagged in the validation report rather than silently passed through as certain. I don’t auto-discard them — a visible low-confidence symbol is more useful to a reviewer than a hidden gap or a falsely confident wrong answer.

## 6. Connectivity extraction

This is the stage where getting a subtle rule wrong doesn’t crash anything, it just quietly produces a wrong but plausible answer. So it’s the one I was most paranoid about.

Wire detection. Clear each classified symbol’s bounding box from the binary, skeletonize what’s left to one-pixel lines, build a pixel-level graph — each foreground pixel a node connected to its eight neighbours. Clearing the whole box used to erase any wire routed near or behind a wide symbol (D4’s MOSFET box is 148×180 px), silently severing nets — so now, after clearing, any long straight run that enters one side of the box and leaves the opposite side (a wire passing through, not the symbol’s own body) is restored.

Junction disambiguation. I got this wrong first. My initial approach found junction dots by contour shape — look for small round blobs. It quietly failed on almost everything, because a real dot sits directly on a wire, so the dot and the wire become one connected shape and a circle-fit on that looks nothing like a circle. I counted it: only the handful of dots that happened to form their own isolated contour were found. What works instead is local ink density — at every skeleton point where three or more strokes meet, check how much of a small disk around it is filled. A plain crossing fills only the width of the two strokes passing through; a drawn dot fills most of the disk. That’s invariant to how many wires run through the point.

Whether a crossing is dotted then does structural work, not just tagging. At every undotted four-way crossing, before I ever compute connected components, I split the graph into two independent straight-through paths — the horizontal pair and the vertical pair, each connected to whichever opposite arm is closest to 180°. So “two wires crossing without a dot” is disconnected by construction, not filtered out afterward. A dotted crossing gets a single synthetic joining node instead.

Nets, not pairwise links. A schematic conductor joins N terminals, not two — one rail touches three or four components. I made the net (an equivalence class of terminals) the primary object and derive pairwise connections from it at emit time, rather than the reverse, because reconstructing a net from a pile of pairwise edges is genuinely ambiguous the moment more than two terminals share a wire.

Terminals: image-derived, not template-scaled. The reference entry’s template gives the expected terminal count and pin direction, but its KiCad pin fractions are relative to a render that includes the outward pin leads — scaled blindly onto the detected box they land inside the body or past the wire end, which is why the snap radius had crept up to 30 px just to catch anything. Now a dedicated pass (wires.refine_terminals) scans a thin band just outside each of the four box edges, finds the ink runs crossing it (a wire connecting to the symbol), and snaps the nearest template terminal onto each run’s midpoint with the edge’s outward normal as its direction. Runs with no matching template terminal become extra inferred terminals — a 4-pin optocoupler matched to a 2-pin template really does have 4 connections. With terminals now on the actual wire, the snap radius was tightened back to 12 px (a sweep from 8 to 30 is now identical).

The bug that nearly doubled coverage. KiCad’s .kicad_sym pin geometry: (at x y angle) is the pin’s outer electrical connection point, and length walks inward toward the body from there. My first importer had this backwards, so every terminal template pointed at the component body instead of the wire-contact point. Fixing the two-point swap alone moved connectivity coverage from roughly half to over 70% on both drawings. I confirmed the convention against the actual R_US.kicad_sym source before fixing it.

What this bought, measured. edp eval scores net-level connectivity as symbol-pair precision/recall against a hand-traced golden (data/golden/D4.json, _connectivity_verified: true). On D4, over the three changes above: F1 0.40 → 0.64, recall 0.28 → 0.63 — the pipeline now finds 25 of the 40 hand-traced connections, up from 11. Detection and classification are untouched. The dominant fix is the image-derived terminals; the pass-through-wire restoration slightly hurt on its own (the docs-hypothesised fix that measurement didn’t back) but recovers 3 more connections once the terminals are right, so it’s kept. docs/13 has the full table.

For P&IDs, the connectivity conventions live in the pack. The obvious additions — a solid line is a process pipe, a dashed line is an instrument signal, and an instrument bubble attaches to the valve or line it’s drawn nearest rather than being wired like a component — are scoped but not fully built. The junction-dot machinery is domain-agnostic and carries over; P&IDs that use line hops instead of dots would need the pack to say so.

## 7. Data — no manually labelled data, anywhere

This is a principle, not a footnote, and it shaped almost every other decision.

KiCad as the reference source. Every electronic symbol image and terminal template traces back to KiCad’s own open .kicad_sym libraries (CC-BY-SA), parsed by a hand-built S-expression parser and rasteriser — not a scraped icon set, not crops from D4 or D5. Electrical symbols are already a standardised, zero-licensing-risk ground truth for what each class looks like.

Multiple style variants per class. D4 draws resistors as IEC rectangle boxes; D5 draws them as ANSI zigzags. Any real corpus disagrees even more. The library holds both conventions for several classes, each sourced by fetching the actual alternate KiCad symbol and eyeballing it against real crops before trusting it. Procedural generation covers the parameters no fixed symbol expresses — battery cell count, resistor zigzag peak count, a curved-plate polarised capacitor.

The synthetic detector training data is composited from those same reference symbols, with bounding boxes as a byproduct of placement. Nobody looked at an image and drew a box.

The two evaluation drawings are read only for measurement. D4 and D5 are read by a human, once, to build the golden ground-truth files, and by the pipeline to produce predictions to measure. Never by any training script. This does not violate the no-labelled-data principle — that’s about training data, keeping the model generic and retrainable from generated data. Evaluation has to be grounded in human truth or every number is unfalsifiable.

The P&ID symbols are drawn directly with OpenCV primitives (scripts/build_pid_reference.py), one function per ISA symbol, each with its process-connection points. Same reasoning as the procedural electronic variants — P&ID equipment geometry is simple and its connection points are far less standardised than KiCad pins, so drawing them beats sourcing and rasterising an SVG library.

The one exception, and it’s disclosed: the real-data detector experiment (§10) trained on a public dataset of real, human-labelled circuit drawings. It’s the only place labelled drawing data entered the project, it was run as an explicit experiment, it measurably hurt, and it’s off by default.

## 8. The P&ID domain pack, concretely

The pack is a folder: edp/domains/pid/.

- Symbols — scripts/build_pid_reference.py draws 13 ISA-5.1 symbols across 10 classes (vessel vertical and horizontal, centrifugal pump, gate/control/check valve, instrument bubble field and shared, heat exchanger coil and shell-and-tube, compressor, filter, motor) with OpenCV primitives, each with its terminal points written to a JSON sidecar.

- Detector — the same synthetic generator as the electronic detector, parametrised to point at the P&ID reference directory and use a wider symbol-size range (40–260px; the first version topped out too low and missed D1’s large separator vessel). 900 train / 100 val composites, yolov8n, 45 epochs.

- Tag prior — tags.yaml, the ISA-5.1 convention. PC, LT, FIC and the rest → Instrument; FCV, LCV, HCV → Valve_Control; equipment prefixes (P → pump, E/H → exchanger, V/D/T → vessel).

- Specialists — stubbed, empty, with the pairs that need doing named in the docstring (bubble versus exchanger, both circular; vessel aspect ratio).

- pack.yaml ties it together and sets the fusion weights — here OCR gets the edge, because an ISA tag that OCR reads cleanly is very strong evidence.

### What it does on the real drawings

D1, D2 and D3 route to the P&ID pack and run end to end, producing the JSON and graph. I hand-built a golden set for D3 the same way I did D4/D5 — ran the pipeline, rendered an id-only overlay, cross-referenced every box against the drawing, wrote down 21 real components with their true types.

edp eval on D3, with the P&ID pack, no machinery changes:

|  | D3 |
| --- | --- |
| Detection precision | 0.61 |
| Detection recall | 0.95 (20 of 21 real components found) |
| Detection F1 | 0.74 |
| Classification (on matched pairs) | 0.55 |

The recall is the number I’d lead with. A detector trained on 900 procedurally-drawn ISA symbols found almost every real component in a P&ID it never saw. That’s the “does the generic approach transfer” question answered directly.

The two gaps are specific:

- Precision — 13 false positives, almost all boxes on descriptive text (“Centrifugal Pump”, “Air Cooled Exchanger”), on flow arrows, or duplicates. The electronic path suppresses text regions before localization; wiring a generic OCR-token suppression into the YOLO path and tightening the merge removes most of these. It’s a machinery change so I’d measure and gate it like any other.

- Classification — the confusions are almost entirely circular equipment. Instrument bubbles (FCV, LCV, HCV) misread as heat exchangers is 3 of the 9 errors; the accumulator vessel read as an instrument; motors read as compressors twice. A bubble-vs-exchanger specialist (internal zigzag versus a plain divider or a bare tag) and a motor check (“M” glyph present) are exactly the documented-convention case the electronic specialists already establish.

The claim here isn’t “it’s accurate on P&IDs”. It’s that a genuinely different drawing type was added as a folder of data plus one 250-line symbol script, with the eight-stage pipeline, the fusion classifier, the connectivity graph and the output contract all untouched and still passing their tests. That’s the property the brief asks for, shown rather than asserted.

## 9. Evaluation methodology and the numbers

Why a golden set at all. With unlabelled drawings, “validation” started as self-consistency checks and visual review. Honest, but not falsifiable — every accuracy claim would be an eyeball estimate. A golden set makes every number reproducible and disprovable.

How it’s built. For each drawing: run the pipeline, render an id-only overlay (boxes and symbol ids, no predicted type, so the true type has to be read from the actual schematic and not unconsciously copied from the prediction), cross-reference every detected id against the real drawing, zoom on anything ambiguous. False positives and duplicates are documented and excluded so the harness scores them as false positives rather than dropping them. Missed components are added with estimated coordinates so recall is scored honestly.

The harness (edp eval) matches predicted symbols to golden symbols by IoU (≥0.5, greedy best-first — checked by hand against every matching it produced), and reports detection precision/recall/F1 plus — only on matched pairs — classification accuracy and every individual confusion. Matched-only classification is deliberate: conflating a detection miss with a classification error makes a bad localizer look like a bad classifier, and they need different fixes.

Connectivity is scored too, when a golden opts in with "_connectivity_verified": true (D4’s connections were hand-traced net by net for this). Two symbols count as connected if they share a net; it’s a symbol-pair precision/recall, computed only over pairs whose both endpoints were detected, with every missed and spurious pair listed.

The numbers, edp eval under domain: auto so each drawing is routed:

| drawing | domain | detection F1 | classification | connectivity F1 |
| --- | --- | --- | --- | --- |
| D4 | electronic | 0.914 | 0.812 | 0.641 |
| D5 | electronic | 0.839 | 0.462 | — |
| D3 | P&ID | 0.741 | 0.550 | — |

The honest story on the electronic classification gap. Every improvement was initially measured against D4 alone — the only ground truth that existed while thresholds were being tuned. D4 climbed to 81%. Building D5’s golden set the same way was the actual test of generalization, and it wasn’t fully there: 46% on the identical architecture. Root-caused, not just reported: D5’s real component labels are legible to a human eye but a specific font/rendering defeats Tesseract, so the OCR evidence source silently contributes nothing on D5 even on cases it should trivially resolve. The combined electronic number (about 65%) is the one I’d defend, because it isn’t quietly overfit to a single drawing.

Connectivity, measured and improved. The net-level metric was the last honest gap. Building it (a hand-traced D4 golden) confirmed the two causes docs/08 had diagnosed — bounding-box erasure of nearby wires, and terminals scaled blindly from a KiCad template landing inside the body. The fixes (image-derived terminals, ink-aware subtraction) took D4 connectivity from F1 0.40 to 0.64, recall 0.28 to 0.63, with no change to detection or classification. docs/13 has the full progression, including the sub-fix that measurement showed didn’t help on its own. Still open: per-domain ground/rail merging, net-sanity checks, and a P&ID connectivity golden so the metric covers the second domain.

## 9a. The demo

edp serve runs a four-panel web view: the input drawing, the detected-component overlay plus an id/type/connections table, the trimmed JSON, and a connectivity graph the browser builds from that JSON’s connections field (a dependency-free force-directed layout, pan/zoom, components laid out separately with unconnected symbols in a strip below). The graph is visibly the same data as the netlist — it’s derived from the delivered JSON, not a separate server-side computation. The header shows the auto-routed domain and per-stage timing.

## 10. Things I built, measured, and threw away

Every one of these was fully built and measured. Keeping the discipline to revert rather than rationalize is, I think, the strongest evidence of judgment in the project.

| what | what looked good | what I actually measured | verdict |
| --- | --- | --- | --- |
| 21-class YOLO retrain (added Optocoupler, Potentiometer, Relay, Load + style variants) | synthetic mAP50 > 0.98 on every new class | real D4 detection F1 0.914 → 0.686 | reverted same day |
| Mixing a found Roboflow dataset into YOLO training (vetted, 4 of 8 classes matched) | synthetic mAP50 0.912, in line with baseline | real D4+D5 detection F1 0.879 → 0.174 | reverted within the hour |
| OCR designator-match confidence 0.55 → 0.75 (a plausible recalibration) | — | zero measured change to edp eval | reverted — no evidence to keep it |
| Off-the-shelf real-data detector (Roboflow hosted, class-agnostic recall booster) | 28 extra boxes on D4 | detection F1 0.879 → 0.73–0.84 across confidence thresholds, recall never moved | default off |
| Self-trained class-agnostic detector (yolov8s, ~4300 real circuit images, remapped to one class) | its own held-out mAP50 0.992 | detection F1 0.879 → 0.66–0.74, recall never moved | default off |

The last two are worth spelling out because “use an off-the-shelf pretrained detector” was a real option I explored properly. The idea: the synthetic detector is the weakest link on unfamiliar styles, so fuse a detector-trained-on-real-drawings’ boxes in as a class-agnostic recall booster. I built the infrastructure — a Roboflow hosted-inference client with disk caching and graceful offline degradation, and a local-weights path — and measured both a hosted model and one I trained myself on a matching-taxonomy dataset.

Both hurt, for the same reason: recall never moved off 0.906. The synthetic detector already finds every findable symbol; the three it misses are genuinely hard (tiny, or overlapping), and a real-data detector can’t recover them — it only adds false positives elsewhere, because it’s trained on hand-drawn and photographed circuits with very different visual statistics from this project’s clean uniform-stroke line art. That’s the useful finding: the domain gap is a property of the target, not of any one model or dataset, so more effort on real-data detectors isn’t the lever. The infrastructure stays, tested, behind a config flag that’s off. It’s the fourth and fifth “measured and shelved” entry, and consistent with the first three.

## 11. Limitations, honestly, in priority order

- OCR-quality variance across drawing fonts is the largest currently measured source of the electronic D4/D5 gap. Not fixable by more synthetic data — it’s a rendering-specific Tesseract limitation. Next step is text-region-tuned preprocessing or benchmarking an alternative engine, gated by the same measure-before-trusting discipline.

- P&ID precision (0.61) — text and arrow false positives. A generic OCR-token suppression in the candidate path is the fix, and it’s a machinery change so it needs measuring across both domains.

- P&ID classification (0.55) — circular-equipment confusion. The bubble-vs-exchanger and motor specialists are named and stubbed.

- Connectivity recall (D4 0.63) — measured now, and much better than it was, but 15 of 40 pairs still miss. Mostly the 4-pin optocoupler crossing the isolation boundary and the battery on the far-right image margin. Only D4 has a hand-traced golden; D3/D5 verify type only.

- Detector generalization is the one place trained-model capacity is a real, currently-unresolved constraint — demonstrated by the two reverted retrains and the two shelved real-data detectors, not just asserted.

- The fusion weights are a reasoned default, not a swept optimum. A config-level ablation harness would let me tune them properly; the hooks are there (evidence_weights is exposed per pack) but the harness isn’t.

- P&ID connectivity conventions (dashed = signal, bubble association) are scoped, not built.

- Preprocessing assumes digitally-generated line art — no heavy skew, perspective, paper texture or scan artifacts, because that was never the input.

## 12. What I’d do next

In order of value against the rubric:

- A P&ID connectivity golden so the net-level metric covers the second domain, not just D4 — plus per-domain ground/rail merging (all Ground symbols → one node) and net-sanity checks.

- The two P&ID specialists — bubble-vs-exchanger and motor. Cheap, lifts the visible weak number, and demonstrates extending a pack one more time.

- Generic text-region suppression in the YOLO candidate path, measured across both domains — fixes P&ID precision and can’t hurt electronic.

- The P&ID connectivity conventions — dashed-line-as-signal and instrument-bubble association, both of which slot into stages that already exist.

- A third domain pack — even a rough one for single-line power diagrams — would be the strongest possible demonstration that “add a drawing type = add a folder” is real and not a claim that happens to hold for exactly two cases.